~Roof Replacement Proposal~

Septembe30, 2016

Marlin Construction, LLC proposes to provide all materials, labor and job management necessary to  complete the following roof systems and roof repairs at 201 NASA Road One  Webster, TX 77598 (see diagram of subject roofs).

Building 1 – Mule-Hide TPO Roof System (60 mil)

- Remove existing TPO roof membrane

- Mechanically attach  tapered ISO to slope roof towards existing internal drain system (see diagram 1)

- Mechanically attach/install 60MIL reinforced TPO Membrane including pre-fabricated accessories

- Install TPO roof jacks

- Install three  4” retro drains

- Fabricate and install new edge metal (Color TBD)

- Remove from property all debris associated with contracted work

- Full 20 Year NDL Warranty for material and workmanship

Price: $23,628.55

Building 2 - Mule-Hide TPO Roof System (60 mil)

- Remove existing modified bitumen roof membrane

- Mechanically attach new tapered ISO to slope roof towards existing internal drains and gutter system (see diagram 2)

- Mechanically attach/install 60MIL reinforced TPO Membrane including pre-fabricated accessories

- Install TPO roof jacks

- Remove and replace (with  new) metal vents

- Install two  4” retro drains

- Fabricate and install  edge metal (Color TBD)

- Fabricate and install  4” gutter and downspouts

- Remove from property all debris associated with contracted work

- Full 20 Year NDL Warranty for material and workmanship

Price: $24,593.96

Building 3 - Mule-Hide TPO Roof System (60 mil)

- Remove existing modified bitumen roof membrane

- Mechanically attach  tapered ISO to slope roof towards existing internal drains and gutter system (see diagram 3)

- Mechanically attach/install 60MIL reinforced TPO Membrane including pre-fabricated accessories

- Install TPO roof jacks

- Remove and replace with all new metal vents

- Install one  4” retro drain

- Fabricate and install new 24 ga edge metal (Color TBD)

- Fabricate and install new 24 ga. 4” gutter and downspouts (Color TBD)

- Remove from property all debris associated with contracted work

- Full 20 Year NDL Warranty for material and workmanship

Price: $35,348.26

Building 4 - Mule-Hide TPO Roof System (60 mil)

- Remove existing TPO roof membrane

- Mechanically attach  tapered ISO to slope roof towards existing internal drains (see diagram 4)

- Mechanically attach/install 60MIL reinforced TPO Membrane including pre-fabricated            accessories

- Install TPO roof jacks

- Remove and replace with all new metal vents

- Install 4 new 4” retro drains

- Fabricate and install new edge metal (Color TBD)

- Remove from property all debris associated with contracted work

- Full 20 Year NDL Warranty for material and workmanship

Price: $35,855.62

Building 5 - Repair

- Remove and replace broken gutter with  24 ga gutter and downspouts

- Repair wall to roof flashing (see photo) - Remove existing  cement fiber board, flash TPO up wall 8”, fabricate and install 24 ga counter flashing

Price: $1,275.00

If contract is executed with all the buildings together Marlin Construction will discount to

Price: $113,676.32

Terms: 30% of contract upon delivery of materials

Balance upon completion

Proposal Expires: October 30, 2016

Acceptance

Date

Print Name and Title:	________________________________________________________________________

Marlin Construction – John Josephs – Sole Member			     Date