James and Webster Presbyterian Committee,

We have put together a proposal for you which we feel is the best strategy for resolving the concerns you have at the Webster Presbyterian Church. The majority of your flat roof buildings have little or no slope and as discussed this presents a problem due to the constant ponding water which is deteriorating the modified bitumen roofs and in turn causing leaks. The ponding water on building 4 is entirely unacceptable in its coverage and depth. This is a very common problem and one which has a solution which we incorporate into our roofing systems quite frequently. What we propose is an approach which leverages poly ISO panels which we mechanically attach to the roof. We will supply your roofing dimensions and proposed sloping to our supplier and they will in turn produce a blueprint for panel locations. These panels are installed in the assigned locations and then we will overlay a reinforced 60 mil TPO. This method results in a roof with slope that moves water to the designated drainage locations.

An additional concern that you expressed involved the internal drains and their ineffectiveness. It would be very expensive to eliminate this system and we have determined that these drains will be effective and sufficient once a tapered roof is in place. We have however seen that the previous roof work has not addressed the drains on the roof. We have included new retro drains in our bid which install and seal into the existing pipes. In locations where we can either add to or improve the gutter and downspout system, we have done so.

Finally, a roof system as proposed above will give you many years of trouble free service and is backed by a No Dollar Limit warranty. This system will add real value to your property and will be installed Marlin Construction which is a local company and we only use professional in-house personnel.

A quick note concerning the cathedral flat roof. This roof is an older TPO roof but was built with taper and appears in generally good condition (it could use a cleaning). There were to things of concern on this and were quoted for repair. The 2 cathedral pictures show these locations. One needs proper TPO flashing repair and the other is a broken gutter with a downspout which is literally dumping water down an expansion joint.

John and I would be happy to sit down with you to answer and address any questions you may have regarding this proposal